样 品 承 认 书

编号:

SAMPLES SPECIFICATION FOR APPROVAL

Switching power Supply/开关电源

客  户:

CUSTOMER:       深圳市贵之族生科技有限公司

产品描述:

DESCRIPTION: Input:100-240V ~ 50-60Hz  Output: DC5V2A

机种型号:

OUR MODEL NO:              J151-0502000CU

样品编号:                              日期:

SAMPLE NO:                     DATE:   2020-04-21

客户编码:                            产品颜色/SAMPLE COLOR:

Customer part number：                Black         White

| APPROVAL SIGNATURE/客户确认 |  |  |
| --- | --- | --- |
| 核准/APPROVED BY | 审核/CHECKED BY | 检测/TESTED BY |

| 深圳市玖洲宝科技有限公司 Manufacturer/制造商 |  |  |
| --- | --- | --- |
| 核准/APPROVED BY | 审核/CHECKED BY | 制定/MAKER |
|  |  | 黄文秀 |

Shenzhen Jiu Zhou Power Technology Co., Ltd.

地址: 中国深圳市宝安区沙井街道和一兴业路金美威工业园第一栋第四层

Add:4thFloor,Block 1,Jinmeiwei Industrial Road,Heyi,Shajing Town,Bao'an District,Shenzhen

Tel: 86-755- 89806019                   Fax: 86-755- 89826202

E-mail: vicky@jzbtech.com              Website：www.jzbtech.com

深圳市玖洲宝科技有限公司 Shenzhen Jiuzhou Power Technology Co., Ltd.

版本目录/Rev. List. 版本/Rev.; 日期/Date; 描述/ Description; 制订/Design. A1; 2020-04-21; 新版本/New Rev.. 目录/Table of contents 1、描述/ DESCRIPTION……………………………………………………………………………………… 3 2、输入特性/INPUT CHARACTERISTICS………………………………………………………………… 3 3、输出特性/OUTPUT CHARACTERISTICS…………………………………………………….…….. …4 4、保护功能/PROTECTION FUNCTION………………………….………………………………………. 5 5、环境要求/ENVIRONMENTAL REQUIREMET……………………………….………………………..5~6 6、安全要求/SAFETY REQUIREMENT………………................................………………..…….…….. 6 7、结构参数/MECHANICAL REQUIREMENT……………………………………..………….…………7 8、外观图OUTLOOK DRAWING …………………. ……….…………. …….………. …….……...…7 9、线材/CORD……………………………………………………………………………………….………..8 10、铭牌附图/LABEL…………………………………………………………...…………. …….………...…9 11、样品测试报告/ SAMPLES ELECTRICAL TEST REPORT…………………………………….... 10

深圳市玖洲宝科技有限公司 Shenzhen Jiuzhou Power Technology Co., Ltd.

1、描述/ DESCRIPTION: 该开关式电源的连续输出功率为 10 W. The S.M.P.S should be capable of a total continuous DC power output of 10 W atts. 2、输入特性/INPUT CHARACTERISTICS: 2.1输入电压/Input Voltage: 额定电压/Rated Voltage: 100-240 Vac 调整范围/Variation Range: 90-264 Vac 2.2输入频率/Input Frequency: 额定频率/ Rated Frequency: 50-60 Hz. 调整频率/Variation Frequency: 47-63 Hz 2.3输入电流/Input Current: 当输入交流电压为额定值的下限电压负载满载时,最大输入交流电流 0.6 A. 0.6 Amps max At any input voltage and rated, DC output rated load. 2.4浪涌电流/Inrush Current: 冲击电流峰值在额定电压输入，冷启动（25℃）时，不大于 30A ;且在任何负载和输入条件下，不导致永久性损坏或危险. Impact current peak in the rated voltage input, cold start (25 ℃), not more than 30 A; And in any load and input conditions, don't cause permanent damage or dangerous. 2.5 AC漏电流/Ac Leakage Current: 当输入电压240Vac时,最大漏电流为0.25mA./ 0.25mA Max.At240Vac input. 2.6 Stand-by power consumption ≤ 0.1 Watts. 待机功耗≤ 0.1 W.

深圳市玖洲宝科技有限公司 Shenzhen Jiuzhou Power Technology Co., Ltd.

3、输出特性/OUTPUT CHARACTERISTICS: 3.1输出功率/Power output 电压 Voltage; 最小负载 Min. Load; 额定负载 Rated. Load; 输出功率 Output power. 5Vdc; 0.01A; 2A; 10W. 3.2输出特性/Output Characteristics 电压 Volt; ge 最小负载 Min. Load; 额定负载 Rated. Load; 电压范围 Voltage Range. 5Vdc; 0.01A; 2A; 空载电压 (V); 负载电压 (V). 4.75-5.35; 4.75-5.35. 3.3纹波和噪音Ripple and Noise: 测试条件:在输入115/230Vac 25℃和输出最小及最大负载时，使用示波器带宽为20MHz连接到适配器的输出端，同时输出端并联一个47uF的电解电容和一个0.1uF的瓷片电容. At 115/230Vac 25℃ input and output Min and Max.Load,the ripple and noise are as follows when measure with Max.Bandwidth of 20MHz and Parallel 47uF/0.1uF,crossed connected at testing point. 电压 最大纹波/最大噪音 Voltage Ripple and Noise(Max.) +5Vdc 120 mV p-p 3.4启动延迟时间/Turn on delay time: 当输入100Vac和输出最大负载时,最大启动时间为 3 S. 3 Second Max.at 100Vac input and output Max.load. 3.5 保持时间/Hold up time: 当输入115Vac 和输出最大负载时,最小保持时间为 5 mS . 5 mS Min.at 115Vac input and output Max.Load. 3.6效率/Efficiency: 当115/230Vac输入电压时，1/4，1/2, 3/4和满载计算平均效率，最小78.7%，适配器满足能效VI. 78.7%Min,At 115/230Vac input voltage,1/4, 1/2, 3/4 and full load calculation average efficiency.Adapter meet efficiency lever VI .

深圳市玖洲宝科技有限公司 Shenzhen Jiuzhou Power Technology Co., Ltd.

4、保护功能/PROTECTION FUNCTION: 4.1短路电路保护/Short circuit protection: 该电源供给器在短路解除时能正常工作恢复. The power supply will be auto recovered when short circuit faults remove. 4.2过流保护/Over current Protection: 过流故障排除后,电源将自动恢复正常工作. The power supply will be auto recovered when over current faults remove. 5、环境要求/ENVIRONMENTAL REQUIREMET: 5.1 工作温度/Operating Temperature: 0℃-35℃,满载,正常工作. 0℃ to 35℃,Full load, Normal operation. 5.2 储藏温度/Storage Temperature:-20℃ to 60℃ 带外壳/With package 5.3工作湿度/Relative Humidity: 5%(0℃)~90%(40℃),72小时,满载,正常工作. 5%(0℃)~90%(40℃)RH,72Hrs,Full load, Normal operating. 5.4 振动/Vibration: 1. 测试标准:国际电工电子委员会 Operating: IEC 721-3-3 3M3 5~9Hz,A=1.5mm 加速度(9~200Hz,Acceleration 5m/S2) 2. 运输/Transportation: IEC 721-3-2 2M2 5-9Hz,A=3.5mm 9~200Hz,加速度Acceleration=5m/S2 200~500Hz,加速度Acceleration=15m/S2 3. 轴向振动/Axes,10 cycles per axis. 在测试过程中不能出现永久性的损坏. No permanent damage may occur during testing. 在电源开启和关闭后,样机能够恢复到最初条件. The SAMPLE has to restore to its original situation after power off/on..

深圳市玖洲宝科技有限公司 Shenzhen Jiuzhou Power Technology Co., Ltd.

5.5 跌落试验/Dropping Packed: 插墙式跌落高度为1米、桌面式跌落高度为760mm、6个面各跌一次 1M for wallmount type and 760mm for desktop type as above described，6 surfaces or one 测试台面是厚13mm的夹木板，离地面高19-20mm The horizontal surface Consists of hardwood at least 13mm thick,mounted on two layers of plywood each 19mm to 20mm thick,all supported on a concrete or equivalent non-resilient floor. 6、安全要求/SAFETY REQUIREMENT: 6.1 主熔断器/ MAIN FUSE 2 Ω 保险电阻 Safety Resistor is 2 Ω 6.2 安全: 符合标准 GB4943 ; Safety:accord standard with GB4943 ; 电磁兼容标准 EMC-Specifcation : GB/T9254-2008; GB17625.1-2012 1. 干扰躁声抑制标准Noise-suppressed acc : IEC55032 ; 2.谐波电流的抑制标准Harmonic current measurement acc : IEC61000-3-2 ; 3.抗静电的承受标准Immunity to electrostatic discharge(ESD)acc:IEC61000-4-2 ; 空气放电Air discharge: ±8KV ; 接触放电Contact discharge: ± 4KV ; 4.抗干扰抑制标准Immunity to radiated electromagnetic field acc:IEC61000-4-3； 测试条件Test characteristic: 80-1000MHz; 80%AM(1KHz) 检验条件Test level 3V/m : A ; 5. 抗快速脉冲群标准 Immunity to fast electic transients(burst)acc: IEC61000-4-4； 耦合方式Coupling On power supplylines 测试条件Test Level：1KV Assessment criteria评估标准: B ; 6.浪涌测试标准 Surge capability acc: IEC61000-4-5； 浪涌电压Surge voltage：1KV Assessment criteria评估标准: B ; 7. 抗骚扰测试Injected currents susceptibility acc: IEC61000-4-6； 测试条件Test Level：3V/m Assessment criteria评估标准: A ; 6.3 高压/DIELECTRIC STRENGTH Hi-Pot: 输入对输出/INPUT to OUTPUT :3000Vac/5mA/60S or 3750Vdc/5mA/60S for type test. （批量生产时/Batch production：3000Vac/5mA/3S） 1.Between input and output <输入（2pin脚短路）---输出（正，负极短路） 2.电压输出变化曲线Voltage time graph

深圳市玖洲宝科技有限公司 Shenzhen Jiuzhou Power Technology Co., Ltd.

6.4 绝缘抗阻/Insulation resistance: 输入对输出/INPUT to OUTPUT 10MΩ min at 500V DC. 7、 结构参数/MECHANICAL REQUIREMENT: (单位：mm) 7.1 外壳/Enclosure: 外壳尺寸: L58.2x W38.7x H26mm; The power supply size: L58.2x W38.7x H26mm; 7.2 输入线/Input Connector: 2 Pin 3C插脚/Two pin input plug of 3C. 7.3 输出/Output USB大四PIN 1正4负, 2、3PIN短路. 8、外观图OUTLOOK DRAWING：mm

深圳市玖洲宝科技有限公司 Shenzhen Jiuzhou Power Technology Co., Ltd.

9、线材/CORD: (单位：mm) 无线材

深圳市玖洲宝科技有限公司 Shenzhen Jiuzhou Power Technology Co., Ltd.

10、铭牌/LABEL: (单位：mm) 居中镭射 灰色字

深圳市玖洲宝科技有限公司 Shenzhen Jiuzhou Power Technology Co., Ltd.

- 样品测试报告/ SAMPLES ELECTRICAL TEST REPORT :